| SAFETY DATA SHEET - EURO-LOCK LOS 300SD Hochtemperatursilicon rot Last Update: 13-01-2005 31.07.2026 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ## SAFETY DATA SHEET |  |  |  |  |  |  |  |
| 1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING |  |  |  |  |  |  |  |
| ##### Product name Supplier Emergency phone number | : : : : | LOS 300 Hochtemperatursilicon rot EURO-LOCK Kamener Str. 33 D – 59368 Werne Tel.: 0049-2389-400400 Fax: 0049-2389-400444 ##### 0049-2389-400400 |  |  |  |  |  |
| 2. COMPOSITION/INFORMATION ON INGREDIENTS |  |  |  |  |  |  |  |
| Chemical characterization | : | Polysiloxane based mixture with fillers, cross linkers and additives. |  |  |  |  |  |
| ###### Hazardous ingredients ≥1% and toxic ingredients ≥ 0,1% | : | #### CAS #### Number | #### Einecs Number | Weight % | Danger Symbol |  | Risk Phrases |
| Ethyltriacetoxysilane |  | #### 17689-77-9 | 241-677-4 | <5 | C |  | 14-34 |
| 3. HAZARDS INDENTIFICATION |  |  |  |  |  |  |  |
| Potential health effects | : | May irritate eyes, respiratory system and skin |  |  |  |  |  |
| 4. FIRST AID MEASURES |  |  |  |  |  |  |  |
| Eyes | : | In case of contact flush eyes immediately with plenty of water for at least 15 minutes. Lift eyelids and rinse well underneath them. Get medical attention if irritation persists. |  |  |  |  |  |
| Mouth (ingestion) | : | Rinse mouth with water several times. Consult a physician and show this Safety Data Sheet. |  |  |  |  |  |
| Lungs (inhalation) | : | Fresh air and rest. Get medical attention if complaint persists. |  |  |  |  |  |
| Skin | : | Remove completely with a dry cloth or paper towel, before washing with detergent and water. Get medical attention if irritation persists. |  |  |  |  |  |
| Clothes | : | Remove polluted clothing. |  |  |  |  |  |
| 5. FIRE-FIGHTING MEASURES |  |  |  |  |  |  |  |
| Suitable extinguishing media | : | CO2, powder, foam and water spray. |  |  |  |  |  |
| Unsuitable extinguishing media | : | Water beam |  |  |  |  |  |
| Special firefighting procedures | : | Wear protective clothing and breathing apparatus. |  |  |  |  |  |
| 6. ACCIDENTAL RELEASE MEASURES |  |  |  |  |  |  |  |
| Environmental precautions | : | Keep away from drains and watercourses. |  |  |  |  |  |
| Clean up procedures | : | Wipe, scrape or soak up in inert material and put in a container for disposal. |  |  |  |  |  |
| Personal precautions | : | Wear proper protective equipment as described in the ‘eye and skin protection’ section of chapter 8. |  |  |  |  |  |
| 7. HANDLING AND STORAGE |  |  |  |  |  |  |  |
| Handling | : | Handle in well-ventilated area. During curing a small amount of acetic acid is emitted. Avoid contact with skin, eyes and do not swallow. |  |  |  |  |  |
| Storage | : | Store in closed packing between 5°C and 25°C away from direct sunlight and other sources of heat. |  |  |  |  |  |
| 8. EXPOSURE CONTROLS/PERSONAL PROTECTION |  |  |  |  |  |  |  |
| Occupational exposure limit | : | MAC-TLV acetic acid (CAS Nr 64-19-7) is 10 ppm, 25mg/m3 |  |  |  |  |  |
| Respiratory protection | : | Avoid breathing the vapor. Maintain good ventilation. |  |  |  |  |  |
| Eye and skin protection | : | Avoid contact with skin and eyes. Wear safety glasses. |  |  |  |  |  |
| ##### Industrial hygiene | : | When using do not eat, drink or smoke. Wash hands after work and before breaks. Don’t rub eyes with dirty hands. |  |  |  |  |  |

| LOS 300 Hochtemperatursilicon rot | SAFETY DATA SHEET |
| --- | --- |

| 9. PHYSICAL AND CHEMICAL PROPERTIES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Physical state |  |  | : | paste |  |  |  |  |  |  | Flash point |  | : | NA |  | °C |  |
| Colour |  |  | : | red |  |  |  |  |  |  | Lower explosion limit |  | : | NA |  | vol-% |  |
| Odour |  |  | : | acetic acid |  |  |  |  |  |  | Upper explosion limit |  | : | NA |  | vol-% |  |
| Density (water=1) |  |  | : | 1,02 |  |  |  |  | g/ml |  | Ignition temperature |  | : | ± 400 |  | °C |  |
| pH (water=7) |  |  | : | NA |  |  |  |  |  |  | Vapour pressure |  | : | NA |  | mbar |  |
| Boiling point (water =100) |  |  | : | NA |  |  |  |  | °C |  | Mixable with water |  | : | no |  |  |  |
| Freezing point (water = 0) |  |  | : | NA |  |  |  |  | °C |  | Mixable with organic solvents |  | : | yes |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | VOC-content |  | : | 0 |  | Weight-% |  |
| 10. STABILITY AND REACTIVITY |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Hazardous decomposition products |  |  |  |  |  | : |  |  | Acetic acid during curing |  |  |  |  |  |  |  |  |
| Materials to avoid |  |  |  |  |  | : |  |  | Water, alcohols and bases |  |  |  |  |  |  |  |  |
| Conditions to avoid |  |  |  |  |  | : |  |  | Unknown |  |  |  |  |  |  |  |  |
| 11. TOXICOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Product information |  |  |  |  |  |  | Ethyltriacetoxysilane |  |  | Acetic acid |  |  |  |  |  |  |  |
| Acute oral LD50 (mg/kg) |  |  |  |  | : |  | 2415 (rat) |  |  | 3310 (rat) |  |  |  |  |  |  |  |
| Acute dermal LD50 (mg/kg) |  |  |  |  | : |  | unknown |  |  | 1060 (rabbit) |  |  |  |  |  |  |  |
| Acute inhalation LC50 (mg/l) |  |  |  |  | : |  | unknown |  |  | 160000 (4h) |  |  |  |  |  |  |  |
| Other information | : | unknown |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| For information on hazardous substances see ‘chapter’ 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12. ECOLOGICAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Avoid uncured product will come in the environment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Degradability |  |  |  |  |  | : |  |  | Silicone part not biological degradable |  |  |  |  |  |  |  |  |
| Ecotoxicity |  |  |  |  |  | : |  |  | Unknown |  |  |  |  |  |  |  |  |
| WGK (Germany) |  |  |  |  |  | : |  |  | 1 |  |  |  |  |  |  |  |  |
| 13. DISPOSAL CONSIDERATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dispose in accordance with federal, state and local legislation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14. TRANSPORT INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| By sea according IMDG-Code |  |  |  |  |  | : |  |  | Class: 2, Limited Quantity: lℓ / 30kg. |  |  |  |  |  |  |  |  |
| By land according ADR |  |  |  |  |  | : |  |  | Class: 2, classification code: 5A, Limited Quantity: LQ2. |  |  |  |  |  |  |  |  |
| By air according ICAO/IATA-DGR |  |  |  |  |  | : |  |  | Class: 2. |  |  |  |  |  |  |  |  |
| UN-number |  |  |  |  |  | : |  |  | 1950 |  |  |  |  |  |  |  |  |
| Proper shipping name |  |  |  |  |  | : |  |  | AEROSOLS |  |  |  |  |  |  |  |  |
| 15. REGULATORY INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| To the labeling requirements of EC Directives 67/548/EC and 1999/45/EC: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Danger symbol |  |  |  |  |  | : |  |  | Not Applicable. |  |  |  |  |  |  |  |  |
| Contains |  |  |  |  |  | : |  |  | Not Applicable. |  |  |  |  |  |  |  |  |
| R-phrases |  |  |  |  |  | : |  |  | Not Applicable. |  |  |  |  |  |  |  |  |
| S-phrases |  |  |  |  |  | : |  |  | Not Applicable. |  |  |  |  |  |  |  |  |
| Extra |  |  |  |  |  | : |  |  | Pressurized container: protect from sunlight and do not expose to temperatures exceeding 50°C. Do not pierce or burn, even after use. |  |  |  |  |  |  |  |  |
| For professional user |  |  |  |  |  | : |  |  | Safety Data Sheet available for professional user on request. |  |  |  |  |  |  |  |  |
| 16. OTHER INFORMATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This product safety data sheet conforms to EC Directive 91/155/EEC as well as its amendments including 2001/58/EC. This data is offered in good faith as typical values and not as product specifications. No warranty either expressed or implied, is hereby made. The recommended industrial and safe handling procedures are believed to be generally applicable, each user should review the recommendations in the specific context of the intended use and determine whether they are appropriate. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NA |  |  |  |  |  | : |  | Not Applicable |  |  |  |  |  |  |  |  |  |
| Unk |  |  |  |  |  | : |  | Unknown |  |  |  |  |  |  |  |  |  |
| R14 |  |  |  |  |  | : |  | Reacts violently with water |  |  |  |  |  |  |  |  |  |
| R34 |  |  |  |  |  | : |  | Causes burns |  |  |  |  |  |  |  |  |  |