Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

!1. Identification of the substance/preparation

Name of product

Manufacturer/distributor

Advice

Emergency advice

EURO-LOCK Chain Spray

EURO-LOCK GmbH

Kamener Straße 33, D-59368 Werne

Phone 0049 (0)2389 400 400, Fax 0049 (0)2389 400 4444

E-Mail info@euro-lock.de

Internet www.euro-lock.de

Vertrieb (über Zentrale vermittelt)

Phone 0049 (0)2389 400 400

Zentrale

Phone 0049 (0)2389 400 400

2. Composition/information on ingredients

Chemical characterization

Polyolefines, organic solvents, additives, propane

Hazardous ingredients

CAS-No.               Name

000074-98-6           propane

000110-54-3           n-hexane

064742-49-0           Naphtha (petroleum), hydrotreated light

[% weight]

15 - 25

< 2,5

45 - 55

Classification

F+

F,

R12

R11-38-48/20-51/53-62-

Xn, N    65-67

Xn;       R11-38-51/53-65-67

F; N

Additional advice

Classification and labelling Xn/R65 is not required for preparations in aerosols containing >=10% of

hydrocarbon solvents (s. Directive 96/54/EC,Annex V, No.8.4)

to CAS-No. 64742-49-0 : This material contains < 0.1 Benzene and must not be classified as carcinogenic (nota

P)

Use: Lubricant

R-Phrases see chapter 16

!3. Special hazards information for man and environment

R-phrases

12

38

51/53

67

Extremely flammable.

Irritating to skin.

Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Vapours may cause drowsiness and dizziness.

! Special hazards information for humans and environment

Risk of bursting at temperatures above 50°C

When used formation of explosive gas/air mixtures may be possible

4. First aid measures

General information

Do not leave casualty unattended.

In case of inhalation

In case of inhalation remove the casualty into fresh air and seek medical advice.

In case of skin contact

In case of contact with skin wash off with soap and water.

In case of eye contact

Eye rinsing with water carefully while protecting unhurt eye.

Page 1/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005

Revision                             21.01.2005

EURO-LOCK Chain Spray

In case of ingestion

If swallowed seek medical advice immediately and show the doctor packing or label.

Physician's information / possible symptoms

unknown

Physician's information / possible dangers

Risk of aspiration by vomiting if swallowed, also without vomiting

Treatment (Advice to doctor)

Treat symptoms.

5. Fire-fighting measures

Suitable extinguishing material

Foam

Dry powder

Carbon dioxide

Extinguishing material that may not be used for safety reasons

Full water jet

Special exposure hazards arising from the substance, combustible products or resulting gases

Risk of bursting by fire heat

Special protective equipment for firefighters

Use breathing apparatus with independent air supply (in closed rooms)

Additional information

Cool containers at risk with water spray jet.

6. Accidental release measures

Personal precautions

Ensure adequate ventillation.

Keep away sources of ignition.

Environmental precautions

Product shall not reach the drain or soil.

Methods for cleaning up/taking up

Stem and take up with absorbent material (e.g. sand, peat, earth).

Additional Information

Keep off any handling equipment with engines (source of ignition)

7. Handling and storage

Advice on safe handling

At work do not eat, drink or smoke

Use only in well-ventilated areas.

Advice on protection against fire and explosion

Risk of injuries and fire spreading by bursting caused by fire heat

Requirements for storage rooms and vessels

Observe National and Local regulation

Advice on storage compatibility

no

Further information on storage conditions

Protect from heat and direct solar radiation.

Page 2/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

Storage group

Fire class

2B

C

Information on storage stability

Storage time above 24 months.

!8. Exposure controls / personal protection

Additional advice on system design

see chapter 7.; - no further measures required

! Ingredients with occupational exposure limits to be monitored

CAS-No.              Name

000074-98-6

000110-54-3

64742-49-0

propane

n-hexane

hydrocarbon vapours

Code

MAK

MAK

MAK

[mg/m³]

1800

180

1000

[ml/m³]

1000

50

200

Remark

Y

Additional advice

no

Respiratory protection

Generally not required

required by exceeding the limit values

! Hand protection

If contamination is possible use chemical-resistant gloves made of nitrile (EN 374; permeation time > 480 min))

Eye protection

Generally not required

Skin protection

Generally not required

General protective measures

Do not inhale aerosols

Hygiene measures

Cloths contaminated with product should not be kept in trouser pockets.

9. Physical and chemical properties

Form

aerosol

Data relevant for safety

Value

pH value in delivery state

Acid number

Changes in physical

state solid/fluid

Changes in physical

state gas

Flash point

Flammable solid

Colour

yellowish

Odour

like petrol (gasoline )

Temperature

at

Method

Remark

not applicable

not applicable

not applicable

not applicable

not applicable

not applicable

Page 3/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

Temperature           at

Method

Value

Flammable gas

Combustion temperature

Autoignition

Lower explosion limit

ca. 1 Vol-%

Upper explosion limit

ca. 11 Vol-%

Remark

not applicable

not applicable

not applicable

informations

concern to propellant only

informations concern to propellant only

Vapour pressure

Density

ca. 3600 hPa

ca. 0,678 g/

cm3

20 °C

20 °C

Bulk density

Rel. vapour density

Solubility in water

Solubility/other

Partition coefficient (log

pOW)

Viscosity 1

Viscosit 2

Solvent separation test

Solvent concentration

not applicable

not applicable

insoluble

organic

solvent

not determined

not applicable

not applicable

not applicable

50 - 55 %

Oxidizing properties

not in the sence of the Dangerous Preparations Directives

Explosive properties

When used formation of explosive mixtures may be possible

Additional information

no

10. Stability and reactivity

Conditions to avoid

near ignition sources(sparks, flames, glow)

Materials to avoid

no

Hazardous decomposition products

no

Page 4/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

Page

Remark

not by destinated use

Additional information

no

11. Toxicological information

Acute toxicity/Irritability/Sensitization

Value/Validation

LD 50 acute oral

LD 50 acute dermal

LC 50 acute

inhalation

Irritability skin

Irritability eye

Sensitization skin

Sensitization

respiratory system

Species

Method

Remark

no data available

no data available

no data available

irritant

low

no

no

Subacute Toxicity - Carcinogenicity

Value

Subacute Toxicity

Subchronic Toxicity

Chronic Toxicity

Mutagenicity

Reproduction-

Toxicity

Carcinogenicity

Toxicity test (Additional information)

no

Experiences made from practice

no negative experience

Additional information

no

Species

Method

Validation

no data available

no data available

no data available

no

no

no

12. Ecological information

Page 5/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

Data on elimination (persistence and degradability)

Elimination rate             Method of analysis

Physico-chemical

degradability

Biological

degradability

Degradability

Biological

eliminability

Degradability

Method

Validation

The product doesn't reach sewage by using as destinated

The product doesn't reach sewage by using as destinated

The product doesn't reach sewage by using as destinated

not applicable

not applicable

according to WRMG

Mobility and bioaccumulative potential

The product has not been tested. Because of the product's consistency and low solubility in water bioavailability

is not likely.

Ecotoxicological effects

Value

Species

Method

Fish

Daphnia

Algae

Bacteria

Behaviour in sewage plant

The product doesn't reach sewage by using as destinated

Validation

no data available

no data available

no data available

no data available

Breathing inhibition of activated sludge

Value

EC 50

Method

Remark

not determined

Additional ecological information

Value

OC

COD

BOD

AOX

Method

Remark

not determined

not determined

not determined

no

Contains following heavy metals and compounds of the 76/464/EWG

no

General regulation

VOC = 0.181 kg/can

Page 6/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

13. Disposal considerations

Waste code No.

16 05 04

Name of waste

gases in pressure containers (including halons) containing dangerous

substances

Recommendations for the product

empty cans: material recycling

filled cans: remove in accordance with local regulations

Recommendations for packaging

Material recycling EWC-No. 150104

Recommended cleansing agent

Hydrocarbon solvent

General information

Give only emptied containers to recycling

14. Transport information

Land and inland navigation transport (ADR/RID/GGVS/GGVE/ADNR)

Identification

UN-No.

Class

PG

Remarks

UN 1950 AEROSOLS, 2.1

1950

2 / Classification code 5F

-

Transportation as Limited Quantities (LQ 2); Marking: UN 1950 (rhombe)

Entry in the transportation document: Transportation according chapter3.4 ADR

Marine transport IMDG/GGV See

Proper shipping name

UN-No.

Class

PG

Marine pollutant

EmS-No.

Remarks

AEROSOLS

1950

2

-

No

F-D,S-U

Entry in transportation document: limited quantities

Marks: UN 1950 (rhombe)

Air transport ICAO/IATA

Proper shipping name

UN/ID-No.

Class

PG

Remarks

UN-4G/Y fibre board boxes required

Label: Flammable Gas

Marking: Aerosols, flammable UN 1950

Transport/further informations

ADR Transport Emergency Card: not required

Aerosols, flammable

UN1950

2.1

-

15. Regulatory information

Classification

F+

Xi

N

Extremely flammable

Irritant

Dangerous for the environment

Page 7/8

Safety data-sheet (91/155 EEC)

Printed                               21.01.2005 Revision                             21.01.2005

EURO-LOCK Chain Spray

R-phrases

12

38

51/53

67

S-phrases

23.4

24/25

29/35

51

Extremely flammable.

Irritating to skin.

Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Vapours may cause drowsiness and dizziness.

Do not breathe spray.

Avoid contact with skin and eyes.

Do not empty into drains; dispose of this material and its container in a safe way.

Use only in well-ventilated areas.

Special labelling for certain preparations

Pressurized container: protect from sunlight and do not expose to temperatures exceeding 50°C. Do not pierce

or burn, even after use.

Do not spray on a naked flame or any incandescent material. Keep away from sources of ignition - no smoking.

Keep out of the reach of children.

Without sufficient ventilation formation of exlosive mixtures may be possible.

National regulations

Restriction of occupation

no

Decree for case of interference/         not applicable

remarks

Classification according to VbF/

remarks

German TA LUFT remarks

not applicable

Water hazard class

not applicable

1

german water pollution classification

Other regulations, restrictions and prohibition regulations

no

16. Other information

Training advice

Use the entries of this Material Safety Data Sheet

Recommendend uses and restrictions

Observe care instructions

Further information

R-Phrases from chapter 2 (not the classification of the product):

R 11: Highly flammable

R 12: Extremely flammable

R 67: Vapours may cause drowsiness and dizziness.

R 65: Harmful: may cause lung damage if swallowed.

R 38 :Irritating to skin.

R 51/53: Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

R 48/20: Harmful: danger of serious damage to health by prolonged exposure through inhalation.

R 62: Possible risk of impaired fertility.

Sources of key data used

Material Safety Data Sheets of raw materials

Page 8/8