EC-Material Safety Data Sheet according 91/155/EWG

Date of issue: 31/07/26			Revision date: 27.08.02				Page 1 of 5

1. Chemical product and company identification

Tradename:			LOS 305  Adhesive

Manufacturer/Supplier:		EURO-LOCK

Kamener Str. 33

D – 59368 Werne

Telephone: 			02389/400400	Telefax:		02389/400444

Emergency information:		02389/400400

2. Composition/information on ingredients

Hazardous components:

| CAS-Number | Hazardous components | concentr.-% | Symbols | R-phrases |
| --- | --- | --- | --- | --- |
| 80-62-6 EINECS: 607-035-00-6 | Methyl Methacrylate | 50-80 | F, Xi | 11,36/37/38,43 |
| 79-71-4 | Methacrylic Acid | 1-15 | C | 34 |

3. Possible dangers

Most important hazards.

Highly flammable. Irritating to eyes, irritating to skin and mucous membranes.

Specific hazards:

Inhalation of high vapour concentrations can cause #CNS-depression and narcosis. Macy cause

sensitization by skin contact.

Code no. 4-5 (1993).

4. First-aid measures

General advice:

Avoid contact with skin and eyes. Avoid breathing vapours or spray mist.

Inhalation:

Move to fresh air: Consult a physician after significant exposure.

Ingestion:

Do not induce vormiting. Drink plenty of water. Call a physician immediately.

Skin Contact:

Remove immediately adhering matter. Was off immediately with soap and plenty of water. If skin irritation persists, call a physician.

Eye Contact:

Rinse thoroughly with plenty of water for at least 15 minutes and consult a physician.

EC-Material Safety Data Sheet according 91/155/EWG

LOS 305 Adhesive

Date of issue: 31/07/26									Page 2 of 5

5. Fire-fighting measures

Suitable Extinguishing Media:

foam, Carbon Dioxide (CO2), dry chemical.

Extinguishing media which must not be used for safety reasons:

Do not use a solid water stream as it may scatter and spread fire.

Specific hazards:

In case of fire and/or explosion do not breathe fumes. Flash back possible over considerable distance.

Polymerization can occur. Polymerization is a highly exothermic reaction and may generate sufficient heat cause thermal decomposition and/or rupture containers.

Special protective equipment for firefighters:

Cool containers/tanks with water spray. If area is heavily exposes to fire and if conditions permit, let fire burn itself out since water may increase the area contaminated. In case of fire, wear a self contained breathing apparatus.

Specific methods:

Standard procedure for chemical fires.

6. Accidental release measures

Personal Precautions:

Avoid contact with skin and eyes. Use personal protective equipment. Ensure adequate ventilation. Remove all sources of ignition,

Environmental precautions:

Avoid subsoil penetration. Do not let product enter drains.

Methods for cleaning up:

Dam up, Scrape up, Soak up with inert absorbent material.

7. Handling and storage

Handling:

Use only in area provided with appropriate exhaust ventilation. ‘To avoid ignition of vapours by static electricity discharge, all metal parts of the equipments must be grounded. Do not smoke.

Storage:

Keep containers tightly closed in a cool, well-ventilated place.

8. Exposure controls and personal protection

# Chemical Name		Exposure Limits

Methyl Methacrylate	208 mg/m³

Methacrylic Acid		72 mg/m³

Engineering measures to reduce exposure:

In case of insufficient ventilation wear suitable respiratory equipment.

EC-Material Safety Data Sheet according 91/155/EWG

LOS 305 Adhesive

Date of issue: 31/07/26									Page 3 of 5

Respiratory protection:

In case of insufficient ventilation wear suitable respiratory equipment.

Hand protection:

Solvent-resistant gloves.

Eye protection:

safety glasses, goggles

Skin and body protection:

Wear as appropriate.

Hygiene measures:

Keep away from food, drink and animal feeding stuffs. Wash hands before breaks and immediately after

handling the product. Regular cleaning of equipment, work area and clothing.

9. Physical and chemical data

| Form: | Paste |  |
| --- | --- | --- |
| Appearance/colour: | Off-White |  |
| Odour: | Pungent |  |
| pH (50 g/l H20) | 7 (20°C) |  |
| Boiling point/range: | 101°C |  |
| Flash point | 10,6 °C |  |
| Explosion limits: | lower 2.0 vol. % upper 12.5 vol. % |  |
| Vapour pressure (20°C): | 23 mm Hg |  |
| Relative density (20°C) | lower 0,92 upper 1.05 |  |

10. Stability and reactivity

Stability:		Stable. Polymerization can occur.

Conditions to avoid:	Direct heating, dirt, chemical contamination, sunlight, UV or ionising

radiation. Heating can release vapours which can be ignited.

Materials to avoid:	oxidizing agents, reducing agents, acids and bases.

Hazardous decomposition products:

carbon oxides, smoke.

11. Toxicological information

Acute toxicity:

Effects of breathing high concentrations of vapour may include, bloody urine, damages Central nervous systems,

lungs.

Methacrylic acid:

LD50/oral/rat = 1060 mg/kg

LC50/inhalation/0 h/rat = >1300 ppm

Methyl methacrylate monomer:

LD50/oral/rat = 7872 mg/kg

LC50/inhalation/0 h/rat = 7093 ppm

Local effects:Dermatits

EC-Material Safety Data Sheet according 91/155/EWG

LOS 305 Adhesive

Date of issue: 31/07/26									Page4 of 5

Sensitization:

Repeated or prolonged skin contact may cause allergic reactions with susceptible persons.

Long term toxicity:

Prolonged or repeated exposure may cause injuries to liver, kidneys, lungs, nerves and hereditary factors

12. Ecological information

Mobility:

Bioaccumulation. Do not contaminate water.

Persistence / degradability:

Methyl methacrylate monomer:

Biochemical oxygen demand within 5 days (BOD5) = .14 g – 0.9 g/g

Bioaccumulation:

Methyl methacrylate monomer: LC50/96h/Fathead minnows = 150 ppm,

LC50/96h/bluegill sunfish = 232ppm

13. Waste disposal considerations

Waste from residues / unused products:

In accordance with local and national regulations

Contaminated packaging:

In accordance with local and national regulations.

European waste Catalogue No:

08 04 09

14. Transport

Proper shipping name:		Adhesives

UN-No.				1133

ADR/RID Class:			3

IMO Class:			3

Proper shipping name:		Adhesives

ICAO Class:			3

Proper shipping name:		Adhesives

HI/UN No:			1133

Packing group:			II

UN/ID No.			1133

15. Regulatory information

# Classification according to European directive on classification of hazardous preparations 93/21/EEC

Contains:		METHYL METHACRYLATE

water contaminating – 1

WGH Nr. 154

Symbols:

F Highly flammable

Xi – Irritant

EC-Material Safety Data Sheet according 91/155/EWG

LOS 305 Adhesive

Date of issue: 31/07/26									Page 5 of 5

R-phrases:

R43		May cause sensitisation by skin contact.

R36/37/38	Irritating to eyes, respiratory system and skin.

S-phrases:

S9		Keep container in a well-ventilated place.

S16		Keep away from sources of ignition – No smoking.

S29		Do net empty into drains.

S33		Take precautionary measures against static discharges.

Code no: 4-5 (1993)

Note: This product is not considered Corrosive to the skin in vivo according to EU Directive 67/548.

EEC as amended.

16. Additional information

Recommended use:	Adhesives

Recommended restrictions:

When used in a mixture, read the labels and safety data sheets of all components. All metal parts of the mixing and processing equipment must be earthed.

These information are based on our present state of knowledge. They describe our product in view of the requirements for safety. They do not assure certain properties of the product. A legal relationship is not vested.