EG-Safety-Data Sheet  LOS 206 Polyurethan PU 50

Manufacturer:		EURO-LOCK

Address:			Flöz-Zollverein-Str. 5

D-59368 Werne

Phone:			0049-2389-400400

Fax:			0049-2389-400444

Emergency phone:	0049-2389-400400

Date:	 Last Update:

:  31/07/2026

18.10.02

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND OF THE COMPANY/UNDERTAKING

Product name

Use of the product

Supplier

Emergency phone number

:    LOS 206 Polyurethan PU 50

:    Sealant for the building industry.

:    See above

:    +49 (0)2389-400400

2. COMPOSITION/INFORMATION ON INGREDIENTS

Chemical characterization                         :    Mixture based on polyurethane prepolymer.

Hazardous ingredients =1%                      :                        CAS

and toxic ingredients = 0,1%

Calcium oxide

Solvent naphtha (petroleum), light aromatic Diphenylmethane-4,4'-di-isocyanate (MDI)

3. HAZARDS INDENTIFICATION

Potential health effects

Number

1305-78-8

64742-95-6

101-68-8

Einecs

Number

215-138-9

265-199-0

202-966-0

Weight

%

1-2,5

2,5-10

1-5

Danger

Symbol

Xi

Xn, N

Xn

Risk

Phrases

37/38-41

10-37-51/53-65-66

20-36/37/38-42/43

:    May cause sensitisation by inhalation and skin contact.

Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

4. FIRST AID MEASURES

Eyes

Mouth (ingestion)

Lungs (inhalation)

Skin

Clothes

5. FIRE-FIGHTING MEASURES

Suitable extinguishing media Unsuitable extinguishing media Special fire fighting procedures

:    In case of contact flush eyes immediately with plenty of water for at least 15 minutes.

Lift eyelids and rinse well underneath them. Get medical attention if irritation persists.

:    Rinse mouth with water several times. Consult a physician and show this Safety Data Sheet.

:    Fresh air and rest. Get medical attention if complaint persists.

:    Remove completely with a dry cloth or paper towel, before washing with detergent and water.

Get medical attention if irritation persists.

:    Remove polluted clothing.

:    CO2, powder, foam and water spray.

:    Unknown.

:    Wear protective clothing and breathing apparatus.

6. ACCIDENTAL RELEASE MEASURES

Environmental precautions

Clean up procedures

Personal precautions

7. HANDLING AND STORAGE

Handling

Storage

:    Keep away from drains and watercourses.

:    Wipe, scrape or soak up in inert material and put in a container for disposal.

:    Wear proper protective equipment as described in the 'eye and skin protection' section of chapter 8.

:    Handle in well-ventilated area. Avoid contact with skin, eyes and do not swallow.

:    Store in closed packing between 5°C and 25°C away from direct sunlight and other sources of heat.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Occupational exposure limit                      :    MAC-TGG (8h)

Respiratory protection

Eye and skin protection Industrial hygiene

Solvent naphtha (petroleum), light aromatic:

Diphenylmethane-4,4'-di-isocyanate (MDI):

50

0,01

ppm

ppm

:    Avoid breathing the vapour. Maintain good ventilation.

:    Avoid contact with skin and eyes. Wear safety glasses.

:    When using do not eat, drink or smoke. Wash hands after work and before breaks.

Don't rub eyes with dirty hands.

page 1/2

EURO-LOCK LOS 206

Polyurethan PU 50

9. PHYSICAL AND CHEMICAL PROPERTIES

Physical state

Colour

Odour

Density (water=1)

pH (water=7)

Boiling point (water =100) Freezing point (water = 0)

:    paste

:    miscellaneous

:    hydrocarbon

:    1,15-1,19

:    Unk

:    Unk

:    Unk

g/ml

°C

°C

SAFETY DATA SHEET

Flash point

Lower explosion limit

Upper explosion limit

Ignition temperature

Vapour pressure

Mixable with water

Mixable with organic solvents

VOC-content

:    >55

:    Unk

:    Unk

:    Unk

:    Unk

:    no

:    partly :    7-8

°C

vol-%

vol-%

°C

mbar

Weight-%

10. STABILITY AND REACTIVITY

Hazardous decomposition products     :    In case of fire: isocyanate, CO, CO2, NOx, HCl, HCN, SO2.

Materials to avoid

Conditions to avoid

:    Unknown.

:    Unknown.

11. TOXICOLOGICAL INFORMATION

Product information

Acute oral LD50 (mg/kg)

Acute dermal LD50 (mg/kg) Acute inhalation LC50 (mg/l) Other information:

:

:

:

Diphenylmethane-4,4'-di-isocyanate

>15000

unknown

0,49

Unknown.

For information on hazardous substances see 'chapter' 2.

12. ECOLOGICAL INFORMATION

Avoid uncured product will come in the environment.

Degradability

Ecotoxicity

WGK (Germany)

13. DISPOSAL CONSIDERATIONS

:    Unknown.

:    Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

:    Unknown.

Dispose in accordance with federal, state and local legislation.

Waste code (Germany)

14. TRANSPORT INFORMATION

By sea according IMDG-Code

By land according ADR

:    080409

:    Not applicable.

:    Not applicable.

By air according ICAO/IATA-DGR     :    Not applicable.

UN-number

Proper shipping name

:    Not applicable.

:    Not applicable.

15. REGULATORY INFORMATION

To the labelling requirements of EC Directives 67/548/EC and 1999/45/EC:

Danger symbol

Contains

R-phrases

S-phrases

:    Xn Harmful

:    Diphenylmethane-4,4'-di-isocyanate

:    42/43     May cause sensitisation by inhalation and skin contact.

52/53     Harmful to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

:    2

23

Keep out of reach of children.

Do not breathe vapour.

36/37     Wear suitable protective clothing and gloves.

45

46

63

In case of accident or if you feel unwell, seek medical advice immediately (show the label where

possible).

If swallowed, seek medical advice immediately and show this container or label.

In case of accident by inhalation: remove casualty to fresh air and keep at rest.

Extra

For DIY user

16. OTHER INFORMATION

:    Contains isocyanates. See information supplied by the manufacturer.

:    Tactile warning applicable.

This product safety data sheet conforms to EC Directive 91/155/EEC as well as its amendments including 2001/58/EC.

This data is offered in good faith as typical values and not as product specifications. No warranty either expressed or implied, is hereby made. The

recommended industrial and safe handling procedures are believed to be generally applicable, each user should review the recommendations

in the specific context of the intended use and determine whether they are appropriate.

NA

Unk

:    Not Applicable

:    Unknown

page 2/2