Seite 1/4

- PRODUCT AND COMPANY INFORMATION

| Product Name: | E 610 Cyanoacrylate |
| --- | --- |
| Manufacturer: | EURO-LOCK |
| Address: | Flöz-Zollverein-Str. 5 D – 59368 Werne |
| Phone Number: | +49 (0) 2389 400400 |
| Fax Number: | +49 (0) 2389 400444 |
| Emergency Phone Number | +49 (0) 2389 400400 |

- Composition / Information Ingredients

| Component Name | CAS | EINEC | Concentration |
| --- | --- | --- | --- |
| Ethyl-2-cyanoacrylate | 7085-85-0 | 230-391-5 | 80 – 100 % |

- Hazards Identification

Classification			Xi	IRRITANT

Irritating to Eyes, respiratory system and skin.

Bonds Skin and Eyes in seconds, highly reactive to water.

- First Aid Measures

First Aid – Inhalation

Remove affected person to fresh air, and if still feeling unwell seek medical attention.

First Aid – Skin

Do not pull bonded skin apart. It may be gently peeled apart using a blunt object such as a spoon, preferably after soaking in warm soapy water. Cyanoacrylates give off heat on solidification. In rare cases a large drop will generate enough heat to cause a burn. Burns should be treated normally after the adhesive has been removed from the skin.

If lips are accidentally stuck together apply warm water to the lips and encourage maximum wetting and pressure from saliva inside the mouth. Peel or roll lips apart. Do not try to pull the lips apart with direct opposing action.

First Aid – Eyes

If the eye is bonded closed, release eyelashes with warm water by covering with wet pad. Cyanoacrylate will bond to eye protein and will cause periods of weeping which will help to debond the adhesive. Keep eye covered until debonding is complete, usually within 1-3 days. Do not force eye open. Medical advice should be sought in case solid particles of cyanoacrylate trapped behind the eyelid cause any abrasive damage.

First Aid – Ingestion

Ensure that breathing passages are not obstructed. The product will polymerise immediately in the mouth making it almost impossible to swallow. Saliva will slowly separate the solidified product from the mouth (several hours).

- Fire Fighting Measures

Non flammable product (flash point is greater than 80°C). If product is involved in fire extinguish with dry powder, foam or carbon dioxide. Trace amounts of toxic fumes may be released on incineration and the use of breathing apparatus is recommended.

Seite 2/4

6. Accidental Release Measures

Ventilate area. Do not use cloths for mopping up. Polymerise with water and scrape off floor.

- Handling and Storage

Handling

Ventilation (low level) is recommended when using large volumes. Use of dispensing equipment is recommended to minimise the risk of skin or eye contact.

Ventilation

For optimum shelf life store in original containers under refrigerated conditions at 2°C to 8°C.

- Exposure Control / Personal Protection

Exposure Limits – UK

Source		Type		Value

OEL		STEL		1.5mg.m-3, 15 minute exposure

Polyethylene or polypropylene gloves are recommended when using large volumes. Do not use PVC, rubber, nylon or cotton gloves. Eye protection should be used where there is any risk of splashing.

- Physical and Chemical Properties

| Physical State | Liquid |
| --- | --- |
| Colour | Clear, Colourless |
| Odour | Sharp. Characteristic |
| pH | N/A |
| Boiling Point / Range | 150 °C |
| Flash Point | 85 °C |
| Specific Gravity | 1.04 g/cm³ |
| Vapour Pressure | 0.293 mmHg |
| Solubility in Water | Insoluble, reacts in water |
| Solubility in Acetone | Completely soluble |
| Explosion Limits (%) | N/A |

- Reactivity and Stability

Polymerisation will occur in the presence of moisture and other basic materials.

- Toxicological information

Inhalation

In dry atmosphere with < 50% humidity, vapours may irritate the eyes and respiratory system. Prolonged exposure to high concentrations of vapours may lead to chronic effects in sensitive individuals.

Skin

Bonds skin in seconds. Considered to be of low toxicity: acute dermal LD50 (rabbit)>2000mg/kg. Due to polymerisation at the skin surface allergic reaction is not considered possible.

Seite 3/4

Eyes

Liquid product will bond eyelids. In a dry atmosphere (RH<50%) vapours may cause irritation and lachrymatory effect.

Ingestion

Cyanoacrylates are considered to have relatively low toxicity. Acute oral LD50 is >5000mg/kg (rat). It is almost impossible to swallow as it rapidly polymerises in the mouth.

- Ecological Information

Biodegradable product of low ecotoxicity. Does not contain any of the substances listed in RoHS and WEEE Directives.

- Disposal Considerations

Dispose of in accordance with local and national regulations. Polymerise by adding slowly to water (10:1). Dispose of as water insoluble non-toxic solid chemical in authorised landfill or incinerate under controlled conditions.

- Transportation Information

Not considered hazardous for the purposes of transportation.

- Regulatory Information

| General | Cyanoacrylate Danger Bonds skin and eyes in seconds. Keep out of the reach of children. |
| --- | --- |
| Labelling Information |  irritant |
| R phrases | R36/37/38 Irritating to eyes, respiratory system and skin |
| S phrases | S23 Do not breath vapour S24/25 Avoid contact with skin and eyes S26 In case of contact with eyes, rinse immediately with plenty of water and seek medical advice. |

- Other Information

The information contained within this MSDS applies only to the product to which the sheet relates. The information provided is based on our best knowledge at the time of issue.

The information contained within this MSDS is believed to be accurate and is given in good faith. However no warranty is made, either express or implied, regarding its accuracy or any liability arising out of the use of the information herein or the products supplied.

When used in other preparations, formulations, or in mixtures, it is necessary to ascertain whether the classification of the hazards have changed. The attention of the user is drawn to the possibility of creating

other hazards when the product is used for purposes other than that for which it was recommended. In such cases a reassessment may be necessary and should be made by the user.

This safety data sheet should only be used and reproduced in order that the necessary measures are taken relating to the protection of health and safety at work.

Seite 4/4

It is the responsibility of the handlers to pass on the totality of the information contained within this document to any subsequent person(s) who will come in to contact with, handle or use this product in any way.

They should check the adequacy of the information provided within this MSDS before passing it on to their customers / staff.

END OF DOCUMENT