EC-Material Safety Data Sheet according 91/155/EWG

Date of issue: 31/07/26			Revision date: 24.01.05				Page 1 of 4

1. Chemical product and company identification

Tradename:			LEAK DETECTION SPRAY

Manufacturer/Supplier:		EURO-LOCK

Kamener Str. 33

D – 59368 Werne

Telephone: 			02389/400400	Telefax:		02389/400444

Emergency information:		02389/400400

2. Composition/information on ingredients

Chemical characteristics:

Leakfinder Spray based on a mixture of watery tensid solvent.

Propellant: Carbon dioxide

Hazardous components:

| CAS-Number | Hazardous components | concentr.-% | Symbols | R-phrases |
| --- | --- | --- | --- | --- |
| 00124-38-9 | Carbon dioxide | 0-5 |  |  |

3. Possible dangers

-.-

4. First-aid measures

Inhalation:

Ensure supply of fresh air. Use mouth-to-mouth or mouth-to-nose resuscitation.

Eye contact:

Rinse thoroughly with plenty of water at open eye lid and seek medical advice.

Skin contact:

Wash with plenty of water and soap. After that moisture the skin.

Further information:

After swallowing immediately consult a doctor for medical advise.

5. Fire-fighting measures

Suitable extinguishing media: without reservations.

^

EC-Material Safety Data Sheet according 91/155/EWG

LEAK DETECTION SPRAY

Date of issue: 31/07/26									Page 2 of 4

6. Accidental release measures

Environmental measurements: Prevent contamination of soil and water. Prevent from spreading or entering into drains, ditches or rivers by using sand, earth or other appropriate barriers

Cleaning-up method:

Absorb or contain liquid with sand, earth or spill control material. Sort out leaking cans and dispose off.

7. Handling and storage

Handling:

Use only in well-ventilated areas. Keep away from sources of ignition. Do not smoke. Do not spray in eyes. Protect against solar radiation.

Storage:

Observe storage regulations for aerosols (in Germany TRG 300). Store in well-ventilated areas only. Storage temperature never above 50°C . Do not carry in car-interior.

8. Exposure controls and personal protection

Contains Carbon dioxide (MAC-value 5000 ppm, 8 hours-mean).

Respiratory protection:

-.-

Eye protection:

Wear goggles.

Skin protection:

Wear protective gloves made of butyl rubber (0,5 mm);breakthrough time >= 8 hours

Industrial hygiene:

After work-time and during work intervals the affected skin areas must be thoroughly cleaned.

At work do not eat, drink, smoke or take drugs.

9. Physical and chemical data

| Physical state | Aerosol |  |
| --- | --- | --- |
| Colour: | Transparent-milk like |  |
| Odour: | Mild |  |
| Change of physical state: | Not applicable |  |
| Flash point: | Not applicable |  |
| Ignition point (spray mist) | Not applicable |  |
| Explosion limits | Lower: no data available upper: no data available |  |
| Vapour pressure [mbar]: | 20°C: 5,5 50°C: 8,0 |  |
| Density at 20°C [g/ml]: | 1,02 |  |
| solubility: | soluble at water |  |
| pH-value 20°C | 6,5 |  |
| Viscosity (mpas) at 20°C | No data available |  |

EC-Material Safety Data Sheet according 91/155/EWG

LEAK DETECTION SPRAY

Date of issue: 31/07/26									Page 3 of 4

10. Stability and reactivity

Thermal decomposition :

None at appropriate use.

Hazardous decomposition products

None at appropriate use.

Hazardous reactions:

At temperatures above 50°C danger of exploding cans.

11. Toxicological information

Contains Carbon dioxide (MAC-value 5000 ppm, 8 hours-mean).

12. Ecological information

Water hazard class: 2 (self-classification)

Watercontaminating. Do not allow to enter soil, waterways or waste water.

13. Waste disposal considerations

For recycling, consult waste disposal centres.

EAK-Number:		16 05 04

14. Transport

ADR/RID:

Name:			Aerosols

UN-Number: 1950	Class: 2		Classification code: 5 A

Limited Quantities: LQ2	Danger label: 2.2

IMGD-code:

Name:			Aerosols

UN-Number: 1950	Class: 2

EmS-Nr.: F-D, S-U		Marine pollutant: no

ICAO/IATA-DGR:

Name:			Aerosols, non-flammable

UN-Number: 1950	Class: 2.2

EC-Material Safety Data Sheet according 91/155/EWG

LEAK DETECTION SPRAY

Date of issue: 31/07/26									Page 4 of 4

15. Regulatory information

The product is classified and labelled in accordance with EC directives / German regulations on dangerous substances.

Symbol:  -.-

Dangers:

Pressurised container. Protect from sunlight and do not expose to temperatures exceeding 50°C. Do not pierce or burn, even after use. Keep out of the reach of children. For proper disposal empty aerosol can completely.

16. Additional information

Duplication without changes of the original version is allowed.

Text for R-phrases shown in section 2 describing each ingredient:

--

These informations are based on our present state of knowledge. They describe our product in view of the requirements for safety. They do not assure certain properties of the product. A legal relationship is not vested.