Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

1. Identification of the substance/preparation

Name of product

Manufacturer/distributor

Advice

Emergency advice

EUROLOCK LOS 350 Sprühkleber Spray

EURO-LOCK GmbH

Flöz-Zollverein-Str. 5, D-59368 Werne

Phone 0049 (0)2389 400 400, Fax 0049 (0)2389 400 4444

E-Mail info@euro-lock.de

Internet www.euro-lock.de

Phone 0049 (0)2389 400 400

Zentrale

Phone 0049 (0)2389 400 400

2. Composition/information on ingredients

Chemical characterization

styrene-butadiene-copolymer, aliphatic hydrocarbons, dimethylether

Hazardous ingredients

CAS-No.               Name

000110-54-3           n-hexane

000110-82-7           cyclohexane

000115-10-6           dimethyl-ether

064742-49-0           Naphtha (petroleum), hydrotreated light

[% weight]

< 2

< 2

30 - 40

40 - 55

Classification

F,         R11-38-48/20-51/53-62-

Xn, N    65-67

F,

Xn, N

F+

Xn;

F; N

R11-38-50/53-65-67

R12

R11-38-51/53-65-67

Additional advice

Classification and labelling Xn/R65 is not required for preparations in aerosols containing >=10% of hydrocarbon solvents (s. Directive 96/54/EC,Annex V, No.8.4)

Use: adhesive

R-Phrases see chapter 16

3. Special hazards information for man and environment

R-phrases

12

38

51/53

67

Extremely flammable.

Irritating to skin.

Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Vapours may cause drowsiness and dizziness.

Special hazards information for humans and environment

Risk of bursting at temperatures above 50°C

Preparation is classified as hazardous in the sense of directive 1999/45/EC.

When used formation of explosive gas/air mixtures may be possible

4. First aid measures

General information

Take affected person into fresh air.

In case of inhalation

In case of inhalation remove the casualty into fresh air and seek medical advice.

In case of skin contact

In case of contact with skin wash off immediately with soap and water.

Page 1/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006

Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

In case of eye contact

Eye rinsing with water carefully while protecting unhurt eye.

In case of ingestion

Do not induce vomiting.

Seek medical advice immediately.

Rinse out mouth thoroughly with water.

Physician's information / possible symptoms

unknown

Physician's information / possible dangers

Risk of aspiration by vomiting if swallowed, also without vomiting

Treatment (Advice to doctor)

Treat symptoms.

5. Fire-fighting measures

Suitable extinguishing material

Foam

Dry powder

Carbon dioxide

Extinguishing material that may not be used for safety reasons

Full water jet

Special exposure hazards arising from the substance, combustible products or resulting gases

Risk of bursting by fire heat

Special protective equipment for fire-fighters

Use breathing apparatus with independent air supply.

Additional information

Remove from fire range, otherwise cool with water spray mist to prevent from bursting by fire heat.

6. Accidental release measures

Personal precautions

Ensure adequate ventilation.

Keep away sources of ignition.

Environmental precautions

Product shall not reach the drain or soil.

Methods for cleaning up/taking up

Stem and take up with absorbent material (e.g. sand, peat, earth).

Additional Information

Keep off any handling equipment with engines (source of ignition)

7. Handling and storage

Advice on safe handling

Use only in well-ventilated areas.

Advice on protection against fire and explosion

Risk of injuries and fire spreading by bursting caused by fire heat

Requirements for storage rooms and vessels

Observe National and Local regulations

Advice on storage compatibility

no

Page 2/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

Further information on storage conditions

Protect from direct solar radiation.

Storage group

Fire class

2B

C

Information on storage stability

Storage time: 24 months.

! 8. Exposure controls / personal protection

Additional advice on system design

no

Ingredients with occupational exposure limits to be monitored

CAS-No.              Name

000110-54-3

000110-82-7

000115-10-6

64742-49-0

n-hexane

cyclohexane

dimethyl-ether hydrocarbon vapours

Code

MAK

MAK

MAK

MAK

[mg/m³]

180

700

1900

1000

[ml/m³]

50

200

1000

200

Remark

Y

Respiratory protection

Generally not required

required by exceeding the limit values

Short term: filter apparatus, filter A

! Hand protection

If contamination is possible use chemical-resistant gloves made of nitrile (EN 374; permeation time > 480 min))

Eye protection

Generally not required

Skin protection

Generally not required

General protective measures

Do not inhale aerosols

Hygiene measures

Cloths contaminated with product should not be kept in trouser pockets.

At work do not eat, drink, smoke or take drugs.

9. Physical and chemical properties

Form

aerosol

Data relevant for safety

Value

pH value in delivery state

Acid number

Changes in physical

state solid/fluid

Changes in physical

state gas

Colour

yellowish

Odour

like petrol (gasoline )

Temperature

at

Method

Remark

not applicable

not applicable

not applicable

not applicable

Page 3/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

Temperature           at

Method

Value

Flash point

Flammable solid

Flammable gas

Combustion temperature

Autoignition

Lower explosion limit

ca. 3 Vol-%

Upper explosion limit

ca. 18,6 Vol-%

Remark

not applicable

for aerosol

cans

not applicable

not applicable

not applicable

not applicable

information

concern to propellant only

information

concern to propellant only

Vapour pressure

ca. 3000 -

4000 hPa

ca. 0,71 g/cm3

20 °C

Density

Bulk density

Rel. vapour density

Solubility in water

Solubility/other

Partition coefficient (log

pOW)

Solvent separation test

Solvent concentration

20 °C

not applicable

not applicable

partially soluble

organic

solvent

not applicable

not applicable

ca. 52 %

Explosive properties

When used formation of explosive mixtures may be possible

Additional information

no

10. Stability and reactivity

Conditions to avoid

near ignition sources(sparks, flames, glow)

Materials to avoid

no

Hazardous decomposition products

not by destinated use

Additional information

no

Page 4/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

11. Toxicological information

Acute toxicity/Irritability/Sensitization

Value/Validation

LD 50 acute oral

LD 50 acute dermal

LC 50 acute

inhalation

Irritability skin

Irritability eye

Sensitization skin

Sensitization

respiratory system

Species

Method

Remark

no data available for the

preparation

no data available for the

preparation

no data available for the

preparation

irritant

low

no

no

Subacute Toxicity - Carcinogenicity

Value

Subacute Toxicity

Subchronic Toxicity

Chronic Toxicity

Mutagenicity

Reproduction-

Toxicity

Carcinogenicity

Toxicity test (Additional information)

no

Experiences made from practice

no negative experience

Additional information

no

Species

Method

Validation

no data available

no data available

no data available

no

no

no

12. Ecological information

Data on elimination (persistence and degradability)

Elimination rate             Method of analysis

Physico-chemical

degradability

Biological

degradability

Method

Validation

The product doesn't reach sewage by using as destinated

The product doesn't reach sewage by using as destinated

Page 5/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

Method of analysis              Method

Validation

Elimination rate

Degradability

Biological

eliminability

Degradability

The product doesn't reach sewage by using as destinated

The product doesn't reach sewage by using as destinated

not applicable

according to WRMG

Mobility and bioaccumulative potential

The product has not been tested. Because of the product's consistency and low solubility in water bioavailability

is not likely.

Ecotoxicological effects

Value

Species

Method

Fish

Daphnia

Algae

Bacteria

Behaviour in sewage plant

The product doesn't reach sewage by using as destinated

Validation

no data available

no data available

no data available

no data available

Breathing inhibition of activated sludge

Value

EC 50

Method

Remark

not determined

Additional ecological information

Value

OC

COD

BOD

AOX

Method

Remark

not determined

not determined

not determined

no

Contains following heavy metals and compounds of the 76/464/EWG

no

General regulation

VOC = 0.244 kg/can

The solvent, which is classified as "toxic for aquatic organism" is fast evaporating and does not reach open

waters.

13. Disposal considerations

Waste code No.

16 05 04

Name of waste

gases in pressure containers (including halons) containing dangerous

substances

Recommendations for the product

empty cans: material recycling

filled cans: remove in accordance with local regulations

Page 6/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

Recommendations for packaging

material recycling EWC 150104

Recommended cleansing agent

Hydrocarbon solvent

General information

Give only emptied containers to recycling

14. Transport information

Land and inland navigation transport (ADR/RID/GGVS/GGVE/ADNR)

Identification

UN-No.

Class

PG

Remarks

UN 1950 AEROSOLS, 2.1

1950

2 / Classification code 5F

-

Transportation as Limited Quantities (LQ 2); Marking: UN 1950 (rhombe)

Entry in the transportation document: Transportation according chapter3.4 ADR

Marine transport IMDG/GGV See

Proper shipping name

UN-No.

Class

PG

Marine pollutant

EmS-No.

Remarks

AEROSOLS

1950

2

-

No

F-D,S-U

Entry in transportation document: limited quantities

Marks: UN 1950 (rhombe)

Air transport ICAO/IATA

Proper shipping name

UN/ID-No.

Class

PG

Remarks

UN-4G/Y fibre board boxes required

Label: Flammable Gas

Marking: Aerosols, flammable UN 1950

Transport/further information

ADR Transport Emergency Card: not required

Aerosols, flammable

UN1950

2.1

-

15. Regulatory information

Classification

F+

Xi

N

Extremely flammable

Irritant

Dangerous for the environment

R-phrases

12

38

51/53

67

Extremely flammable.

Irritating to skin.

Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

Vapours may cause drowsiness and dizziness.

Page 7/8

Safety data-sheet (91/155 EEC)

Printed                               01.06.2006 Revision                             01.06.2006

EUROLOCK Sprühkleber Spray

S-phrases

23.4

24/25

29/35

51

Do not breathe spray.

Avoid contact with skin and eyes.

Do not empty into drains; dispose of this material and its container in a safe way.

Use only in well-ventilated areas.

Special labelling for certain preparations

Pressurized container: protect from sunlight and do not expose to temperatures exceeding 50°C. Do not pierce

or burn, even after use.

Do not spray on a naked flame or any incandescent material. Keep away from sources of ignition - no smoking.

Keep out of the reach of children.

Without sufficient ventilation formation of explosive mixtures may be possible.

National regulations

Restriction of occupation

no

Classification according BetrSichV Extremely flammable

(german regulation)

Water hazard class

1

self-classification to German water polution class

Other regulations, restrictions and prohibition regulations

no

16. Other information

Training advice

Use the entries of this Material Safety Data Sheet

Recommended uses and restrictions

Observe care instructions

Further information

R-Phrases from chapter 2 (not the classification of the product):

R 11: Highly flammable

R 12: Extremely flammable

R 67: Vapours may cause drowsiness and dizziness.

R 65: Harmful: may cause lung damage if swallowed.

R 50/53: Very toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

R 38 :Irritating to skin.

R 51/53: Toxic to aquatic organisms, may cause long-term adverse effects in the aquatic environment.

R 48/20: Harmful: danger of serious damage to health by prolonged exposure through inhalation.

R 62: Possible risk of impaired fertility.

Sources of key data used

Material Safety Data Sheets of raw materials

Page 8/8