Material Safety Data Sheet (91/155/EG)

EURO-LOCK A 62.00 ANAEROBIC SEALANT             31.07.2026    Pag. 1/4

Last Update: 14.07.05

1.1 Product

Trade name:       A 62.00 ANAEROBIC SEALANT

Technical name:   Anaerobic adhesive and sealant

Chemical name:    methacrylate esters and composition

1.2  Company

EURO-LOCK

Kamener Str. 33

D 59368 Werne

02389/400400 Telefax: 400444

2.  Composition/Information on ingredients

Hazardous ingredients           %  Classification R-Phrases

UE N.607.

134.00.4 	Methacrylatester          <5        Xi     36/37/38

27813-02-1	Hydroxypropylmethacrylate 0,1-1     Xi     36,43

EINECS 248-666-3

868-77-9	Hydroxyethylmethacrylate  0,1-2     Xi     36/38,43

EINECS 212-782-2

80-15-9	Cumene hydroperoxide      1         O,T,N

7,21/22/23,

EINECS 201-254-7                           34,48/20/22,

51/53

3.  The Main Hazard

Irritating to eyes, respiratory system and skin, may cause

sensitization by skin contact.

4.  First Aid Procedures

Product on skin:

immediately wash with water and soap. Remove contamined

clothing and wash before re-use. In case irritation persists

consult a doctor.

Product on eyes:

immediately flush the eye with plenty of water for at least 10

minutes, holding eye open. Obtain medical attention urgently.

Product ingested:

wash out mouth with water and give cold water or milk.. Obtain medical attention.

Product inhaled:

expose to fresh air.

5.  Fire Protection

Extinguishing media:

water spray jet, foam, dry chemical, carbon dioxide.

Special extinguishing procedures:

wear self-contained breathing apparatus. Formation of toxic

gases.

Explosion and firing hazard:

keep containers cool with water spray.

Material Safety Data Sheet (91/155/EG)

EURO-LOCK A 62.00 ANAEROBIC SEALANT            31.07.2026    Pag. 2/4

Last Update: 14.07.05

6.  Spillage

Wear protective equipment, ensure adequate ventilation, do

not convoy to sewage. Adsorb spills using earth or sand and

collect for disposal according to regulations.

7.  Handling and storage

Handling			:avoid contact with skin and eyes.

use in well ventilated area.

Storage precautions	:store below 28°C, away from direct

sunlight and heat.

8.  Exposure control/Personal protection

General protective measures:

Do not inhale vapours. Avoid contact with eyes and skin.

Store work clothing separately. Remove soiled or soaked

clothing immediately. Follow the usual good standards of

occupational hygiene. Use skin protective preparation as

preventive skin protection.

Hand protection:

Butyl rubber gloves (0,7 mm), Breakthrough time 300 min

(EN 374), other advisable gloves PE or nitrile.

Frequently in practice there are differing conditions. So

the given considerations can only be an aid to orientation

for the selection of a suitable chemical protection glove.

Particularly they do not compensate for suitability tests

by the end user.

Gloves should be replaced regularly, especially after extended

contact with the product. For each work-place a suitable glove

type has to be selected.

Eye protection:

Tightly fitting goggles.

9.  Physical and chemical data

Odour					: slightly pungent smell

Colour					: green

Change of physical state		: polymerisation over 100°C

Flash point (COC/DIN/ISO2592) : more than 100°C

Explosion limits			: n.a.

Autoignition temperature		: more than 380°C

Vapour pressure(20°C)DIN51616	: <0,5 mbar

Viscosity (Brookfield)(25°C)	: 2500-4000 mPas

Water solubility (20°C)		: not soluble

pH-value					: 4-6

Density (20°C) DIN 51757		: 1,0 – 1,1 g/ml

Material Safety Data Sheet (91/155/EG)

EURO-LOCK A 62.00 ANAEROBIC SEALANT             31.07.2026    Pag. 3/4

Last Update: 14.07.05

10. Stability and reactivity

Product is stable at normal conditions of storage and use.

Conditions to avoid:	heating over 100°C. –

Exposure to direct sunlight.

Materials to avoid:		strong oxidising

agents, metals/rust, strong acids.

Hazardous decomposition products:

burning produces carbon oxide and toxic fumes.

11. Health hazard information

Acute Toxicity:	LD 50 oral > 5000 mg/kg (rat)

for analogy with similar product

Skin:			Irritant

Eyes:			Irritant

Sensitization:		Possible by skin contact

Inhalation:		May irritate the respiratory system.

12. Environmental data

Do not convoy in water discharges.

Hazard class for water: WGK 1 (selfclassification).

slightly hazardous for water

13. Disposal

In accordance with local regulations.

European Waste nr: 080400

wastes from MFSU of adhesives and sealants

14. Transport information

Way				Class			Label

Road-Rail			ADR/RID  :--		---

Sea				IMO/IMDG :--		---

Air				ICAO/IATA:--		---

Not classified. Labelling not required.

15. Labellling information

Labelling as EEC regulations in force:

Hazard symbol: St. Andrew´s Cross – Xi – irritating

Contains: Hydroxypropyl- Hydroxyethylmethacrylate

Risk and saftey phrases:

R 36/37   Irritating to eyes and respiratory system.

R 43		May cause sensitization by skin contact.

s “$”%	Avoid contact to skin and eyes.

S 26      In case of contact with eyes, rinse immediately

with plenty of water and seek medical advice.

S 28      After contact with skin wash immediately

with plenty of water and soap.

S 37/39   Wear suitable gloves and eye/face protection.

Voluntary classification:

R 38		Irritating to skin.

Material Safety Data Sheet (91/155/EG)

EURO-LOCK A 62.00 ANAEROBIC SEALANT             31.07.2026    Pag. 4/4

Last Update: 14.07.05

16. Additional information

The information contained herein is based on our present

state of knowledge and experience and according to

EC Regulations and other related: 91/155(2001/58),

67/548(2001/59), 1999/45(2001/60), 91/689(2001/118)

89/542, ADR 23.072001, IMDG-Code 30° amd. IATA-DRG 2002.

Relevant R-Phrases:

R 21/22	Harmful in contact with skin and if swallowed.

R 23		Toxic by inhalation.

R 34		Causes burns.

R 36		Irritating to eyes.

R 36/37/38 Irritating to eyes, respiratory system and skin.

R 36/38	Irritating to eyes and skin.

R 43		May cause sensitization by skin contact.

R 48/20/22 Harmful, danger of serious damage to health by

prolonged exposure through inhalation and if

swallowed.

R 51/53	Toxic to aquatic organisms, may cause long term

adverse effects in the aquatic environment.

R 7		May cause fire.